- Given an IP address range of 192.168.1.0/24, divide the network into 4 subnets.

Task: Manually calculate the new subnet mask and the range of valid IP addresses for each subnet.

We need 4 subnets. To find how many bits to borrow, we use the formula 2^n >= {number of subnets}.

- 2^2 = 4, so we must borrow 2 bits.

The original mask was /24 (255.255.255.0). Adding the 2 borrowed bits makes it a /26.

- Binary of last octet: 11000000 (which is 128 + 64)

- New Subnet Mask: 255.255.255.192

Step C: Determine the "Magic Number" (Block Size)

Subtract the last non-zero octet from 256:

- 256 - 192 = 64

Every subnet will start in increments of 64.

| Subnet | Network Address | First Valid Host | Last Valid Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 |
| 2 | 192.168.1.64 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 |
| 3 | 192.168.1.128 | 192.168.1.129 | 192.168.1.190 | 192.168.1.191 |
| 4 | 192.168.1.192 | 192.168.1.193 | 192.168.1.254 | 192.168.1.255 |