9) You configured VLANs 10 and 20 on your switch and assigned ports to each VLAN. However, devices in VLAN 10 cannot communicate with devices in VLAN 20, Troubleshoot the issue.

Understand the issue

- VLANs are separate networks → no communication by default

Check VLANs

- Use show vlan brief

- Ensure ports are in correct VLANs

Verify port mode

- Ports must be in access mode

- Correct VLAN assigned

Check IP configuration

- VLAN 10 → 192.168.10.x

- VLAN 20 → 192.168.20.x

Root cause

- No inter-VLAN routing (Layer 2 switch limitation)

Fix

- Use router or Layer 3 switch

- Configure trunk + router subinterfaces

Set default gateway

- VLAN 10 → 192.168.10.1

- VLAN 20 → 192.168.20.1

Test

- Ping within VLAN → then across VLAN