To get 4 subnets, we use the formula $2^n = \text{number of subnets}$, where $n$ is the number of bits borrowed.

- $2^2 = 4$, so we need to borrow 2 bits.

- Original mask: /24 ($255.255.255.0$)

- New mask: $24 + 2 =$ /26

In decimal, those 2 borrowed bits in the last octet ($11000000$) equal $128 + 64 = 192$.

New Subnet Mask: $255.255.255.192$

2. Determine the Valid Host Ranges

With a /26 mask, each subnet has a total of 64 IP addresses ($256 / 4 = 64$). However, we subtract 2 for the Network ID and Broadcast address.

| Subnet | Network ID | First Valid Host | Last Valid Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| 1 | 10.0.0.0 | 10.0.0.1 | 10.0.0.62 | 10.0.0.63 |
| 2 | 10.0.0.64 | 10.0.0.65 | 10.0.0.126 | 10.0.0.127 |
| 3 | 10.0.0.128 | 10.0.0.129 | 10.0.0.190 | 10.0.0.191 |
| 4 | 10.0.0.192 | 10.0.0.193 | 10.0.0.254 | 10.0.0.255 |